HALO OUTBOARDS

BLUETOOTH  FOTA TECHNICAL SPECIFICATION

Version 2.0

Revision History

| Version | Date | Notes |
| --- | --- | --- |
| 1.0 | 06/14/2025 | Initial version |
| 2.0 | 02/11/2026 | Change the frame format to the new standard |

Contents

1. Overview	4

2. Requirement	4

3. FOTA Flow	4

4.  Prerequisites	7

5. Command List	7

6. Command Details	8

6.1 HELLO Command	8

6.2 FW_INFO Command	8

6.3 ERASE Command	8

6.4 WRITE Command	8

6.5 GOTOAPP Command	8

7. Programming Process	9

8. Notes for Developers	9

# 1. Overview

This document defines the mobile application–side requirements for downloading firmware to  device over Bluetooth (BLE or Bluetooth SPP).

# 2. Requirement

For Mobile (Android, Ios) app, the suggest requirement as follow

- Have bluetooth device selection

- Connect button to initiate communication

- Program button to flash the firmware

- Erase button to erase entire chip

- Log console area for status messages

- Progress bar for flashing status

- Support firmware file in .bin or .hex

# 3. FOTA Flow

The process to FOTA the Mainboard will follow through suggested algorithm in the picture:

# 4.  Prerequisites

- Mainboard must be in Bootloader mode by issuing a software command to enter boot mode (hardware way is no longer supported in new hardware version). The command is: “$APP_CONFIG,UPDATE_FW”.

- Bluetooth communication must be configured using one of the following transport modes:

a) Bluetooth Classic (SPP – UART over Bluetooth)

•  Baud rate: 115200

•  Data bits: 8

•  Parity: None

•  Stop bits: 1

b) Bluetooth Low Energy (BLE – UART-like service)

• Custom or Nordic UART–style BLE service

• Data is transferred as raw byte streams

• Packet size depends on negotiated BLE MTU

# 5. Command List

| Command | ID | Direction | Description |
| --- | --- | --- | --- |
| HELLO | 0x79 | App → STM32 | Init command to check connection to bootloader |
| FW_INFO | 0x10 | App → STM32 | Inform bootloader firmware start address and total firmware size |
| ERASE | 0x43 | App → STM32 | Erase old firmware |
| WRITE | 0x31 | App → STM32 | Write Firmware Block by Block |
| GOTOAPP | 0x21 | App → STM32 | Start the new firmware after finish WRITE |
| ACK | 0x79 | STM32 → App | Command/Write success |
| NACK | 0x1F | STM32 → App | Command/Write failure or Write abort(Write cmd only) |

# 6. Command Details

## 6.1 HELLO Command

Purpose: Check connection to bootloader.

Frame description: [CMD_HELLO_ID]|[ACK] or [NACK]

Detail: When App send [CMD_ HELLO_ID], wait bootloader return [ACK] to conclude command is success or not before continue send another command. Stop if receive [NACK] or no respond.

## 6.2 FW_INFO Command

Purpose: Send to bootloader start address and size of firmware file.

Frame description: [CMD_FW_INFO_ID][Start_address][Firmware_size]|[ACK] or [NACK]

Detail: App send total 9 byte:

ID: 1 byte

Start_address: 4bytes

Firmware_size: 4bytes

Handler ACK logic like HELLO Command

To ensure firmware is working , start_address is fixed to 0x08004000

## 6.3 ERASE Command

Purpose: Erase entire application flash area.

Frame description: [CMD_ERASE_ID]|[ACK] or [NACK]

Detail: Check ACK logic like HELLO Command

## 6.4 WRITE Command

Purpose: Write firmware to bootloader, block by block.

Frame description: [CMD_WRITE_ID]|[ACK] or [NACK]|[DATA_BLOCK1] |[ACK] or [NACK]|[DATA_BLOCK2]| |[ACK] or [NACK]….

Detail: After send Write ID and receive ACK, begin send stream of data from firmware file to bootloader in the block of 256 bytes each (DATA_BLOCK). Each send have to wait bootloader finish processing by wait for ACK before send new block, do it until all content of firmware file is over.

## 6.5 GOTOAPP Command

Purpose: Start running new firmware

Frame description: [CMD_GOTOAPP_ID]|[ACK] or [NACK]

Detail: Handler ACK logic like HELLO Command

# 7. Programming Process

Program firmware is start after pressing flash firmware button on app. Procedure as follow:

- Send HELLO command to check the connection

- Send FW_INFO command to let bootloader know the start address and size of firmware.

- Send ERASE command to clean the old firmware.

- Send WRITE command to begin flash firmware to the bootloader.

- Send GOTOAPP command after finish flashing firmware to start the firmware app

# 8. Notes for Developers

- Firmware file come with .bin or .hex format. To ensure content of .hex file is able to send to mainboard, .hex file decoding to binary is require.

- For BLE communication, UART parameters (baud rate, parity, stop bits) are not applicable. These settings are listed only for Bluetooth Classic (SPP) mode.

- Ensure proper ACK handling and timeout logic for reliability

- In all described steps, if a NACK (0x1F) appears, display an error in message log and stop any ongoing process related to programming.

- Provide real-time logs to the UI for transparency

- Implement retry and failure recovery logic for robustness