HALO OUTBOARDS

TILLER BOARD REPORT DATA SPECIFICATION

Version 1.4

Revision History

| Version | Date | Notes |
| --- | --- | --- |
| 1.0 | 06/14/25 | Initial version |
| 1.1 | 09/13/2025 | Change the frame format to the new standard |
| 1.2 | 10/12/2025 | Add wattage parameter to BMS data group |
| 1.3 | 10/23/2025 | 1. Change index of BMS data group 2. Add voltage and power wattage parameter of VESC data group |
| 1.4 | 01/28/2026 | 1. Add INFOR data |

Contents

I.	Overview	4

II.	Data frame description	4

1.	GNSS data	4

2.	BMS data	5

3.	MOTOR data	6

4.	VESC data	7

5.     INFOR data	8

III.	Note for Developer	10

- Overview

This document describe data frame formats used for bluetooth serial comunication between tiller board and another device (like android or other bluetooth type device). The data is transmitted as plain text using comma-separated values and begins with a frame header.

- Data frame description

- GNSS data

Data direction:

- From: Tiller Board

- To: App (Android or similar platform)

- Purpose: Report real-time GNSS (GPS, GLONASS, Baidu….) data

Frame format:

$GNSS, Gx, [data...]

- $: Start of Frame

- GNSS: Data Frame type

- Gx: Message group type, which defines the structure of the remaining data fields.

GNSS Group 1 Format:

$GNSS,G1,<TimeUTC>,<Latitude>,<Longitude>,<Course>,<Speed>

GNSS Group 1 frame description:

| Name | Type | Data example | Description |
| --- | --- | --- | --- |
| TimeUTC | int | 123519 | Time in UTC (hhmmss format), GMT 0 |
| Latitude | float | 10.762622 | Latitude in decimal degrees |
| Longitude | float | 106.660172 | Longitude in decimal degrees |
| Course | float | 219.2 | Course over ground in degrees from North |
| Speed | float | 45.5 | Speed over ground in kilometers per hour (km/h) |

Frame example

$GNSS,G1,123519,10.762622,106.660172,84.2,45.5

- BMS data

Data direction:

- From: Tiller Board

- To: App (Android or similar platform)

- Purpose: Report real-time BMS data

Frame format:

$BMS, Gx, [data...]

- $: Start of Frame

- BMS: Data Frame type

- Gx: Message group type, which defines the structure of the remaining data fields.

BMS Group 1 Format:

$BMS,G1, < Voltage >,< Capacity >,Current>,< Wattage >,< Temperature >

BMS Group 1 frame description:

| Name | Type | Data example | Description |
| --- | --- | --- | --- |
| Voltage | float | 48.7 | Battery voltage in volts (V). |
| Capacity | int | 92 | Battery capacity in percent (%) |
| Current | float | 12.5 | Battery current in amps (A). |
| Wattage | Int | 1460 | Battery power consumption (W) |
| Temperature | Int | 36 | Battery temperature in Celsius (°C) |

Frame example

$BMS,G1,48.7,92, 12.5,1460,36

- MOTOR data

Data direction:

- From: Tiller Board

- To: App (Android or similar platform)

- Purpose: Report real-time Motor data

Frame format:

$MOTOR, Gx, [data...]

- $: Start of Frame

- MOTOR: Data Frame type

- Gx: Message group type, which defines the structure of the remaining data fields.

MOTOR Group 1 Format:

$MOTOR,G1,<Phase Current>,< MotorRPM >,<Temperature>

MOTOR Group 1 Frame description:

| Name | Type | Data example | Description |
| --- | --- | --- | --- |
| Phase Current | float | 16.7 | Motor current in amps (A) |
| MotorRPM | int | 1420 | Motor speed in revolutions per minute (RPM). |
| Temperature | int | 36 | Motor temperature in Celsius (°C). |

Frame example

$MOTOR,G1,16.7,1420,36

- VESC data

Data direction:

- From: Tiller Board

- To: App (Android or similar platform)

- Purpose: Report real-time VESC data

Frame format:

$ VESC, Gx, [data...]

- $: Start of Frame

- VESC: Data Frame type

- Gx: Message group type, which defines the structure of the remaining data fields.

VESC Group 1 Format:

$VESC,G1,<Voltage>,<Current>,<Wattage>,<Throttle>,<Temperature>

VESC Group 1 frame description:

| Name | Type | Data example | Description |
| --- | --- | --- | --- |
| Voltage | float | 48.5 | VESC voltage in volts (V) |
| Current | float | 28.5 | VESC current in amps (A) |
| Wattage | Int | 3280 | VESC power consumption (W) |
| Throttle | int | 50 | Throttle level in percent (%) |
| Temperature | int | 40 | VESC temperature in Celsius (°C). |

Frame example

$VESC,G1,48.5,28.5,3280,50,40

## 5. INFOR data

Data direction:

- From: Tiller Board

- To: App (Android or similar platform)

- Purpose: Report real-time Information data

Frame format:

$ INFOR, Gx, [data...]

- $: Start of Frame

- INFOR: Data Frame type

- Gx: Message group type, which defines the structure of the remaining data fields.

INFOR Group 1 Format:

$INFOR,G1,<Firmware version >,<Serial number>

INFOR Group 1 frame description:

| Name | Type | Data example | Description |
| --- | --- | --- | --- |
| Firmware version | String | 1.0.2 | Firmware version v1.0.2 |
| Serial number | String | BLD2008397 | Serial number |

Frame example

$VESC,G1,1.0.2,BLD2008397

INFOR Group 2 Format:

$INFOR,G2,<Odomerter>,<Driver mode>,<Error>

INFOR Group 2 frame description:

| Name | Type | Data example | Description |
| --- | --- | --- | --- |
| Odometer | float | 1000.0 | km |
| Driver mode | String | Sport | Eco, Normal, Sport, Docking |
| Error | String | E01 | Error code |

Error code table

| Error code | LCD display | Explanation | Cause |
| --- | --- | --- | --- |
| E01 |  | Overvoltage | Battery voltage > 65V |
| E02 |  | Undervoltage | Battery voltage < 42V |
| E03 |  | BMS over temperature | Temperature > 85oC |
| E04 |  | VESC over temperature | Temperature > 85oC |
| E05 |  | Motor over temperature | Temperature > 85oC |
| E06 |  | GPS not found | GPS not found |
| E07 |  | VESC not found | VESC not found |
| E08 |  | BMS not found | BMS not found |

Frame example

$INFOR,G2,1000,Sport,E01

- Note for Developer

- Separator: Fields are separated by commas < , >

- Line Ending: Each frame is sent as a single line (\r\n)

- Suggest parsing: On Android or similar platform, user can get data by splitting string frame by comma < , > after confirming the header prefix (e.g., starts with $GNSS, $BMS….).

- Transmission Protocol: Using bluetooth classic serial communication.